Lab 6

Get Voc’

Tweak Resistance to get Voc/2 and the resistance would be equal to Rth

1) Show   the   mathematical derivation   for   how   we   know   the   voltage   at   two   nodes   will   be Voc/2when a resistor, Rload, is connected to those nodes that is equal to RTh.  Hint: Begin by drawing  the generic  Thevenin  Equivalent  Circuit  with Rload  attached  to  the  two  nodes(similar to Figure 3)and write out the voltage divider equation.

2)  Measure  and  record   the  resistance  between  pin1-pin2  and  between  pin2-pin3  of  your variable resistor.  What value do these sum to?  Measure the resistance between pin1-pin3to check (this value should be close to the previous summation).   Individuals’ pin1-pin2 andpin2-pin3 resistance measurements will vary depending on where your wiper happens to be currently set.  (Everyone’s summation should be close to 10kΩ, though.)

2. data

  - Pin 1 & 2 resistance 2.025 kOhm

  - Pin 2 & 3 resistance 8.0577 kOhm

  - Pin 1 & 3 resistance 9.8577

  - Sum of 1&2 and 2&3 = 10.0877

3. data

- Vs = 15.656v

Data for box 1

- Voc = 5.2121v

- V0c/2 = 2.605v

- Voc/2 mesured = 3.6045

- Voltage drop from b1-b2 =

- Rth = 5.3194 kOhms

Data for box 2

- Voc = 15.617

- Voc/2 = 7.8085

- Voc/2 mesured = 7.8001

- Rth = 1.9938 kOhms

Data for box 3

- Voc = -3.1239

- Voc/2 = -1.56195

- Voc-2 measured = -1.5632

- Rth = 1.204 kOhms

4.

Open box 1

Color code for resistors

Open box 2

Open Box 3

5). Now  knowing  what  you  do  about  this  specific  black  box,  is  it  possible  to  measure  theThevenin  resistance  at  B1-B2  directly  from  the  black  box?    If  so,  explain  how  you  would  doit?   If  not,  why  not?    Hint:  keep  in  mind  that  you  have  access  to  the  only  power  supply“in”the black box (Vs connected at A1-A2).